Mohammed Khider University of Biskra

Faculty of Science and Technology

2nd year License

Module: TP ELN Fund 1

Lab N 01: Electrical measurements

- Objectives of the Lab:

At the end of this practical work, the student will learn the following knowledge:

1. Recognize the various electronic components constituting an electrical circuit;

2. How to take measurements of voltages and electrical currents (and others) in a circuit;

3. Use the multimeter (voltmeter, ohmmeter, etc.), and take safety precautions;

4. Visualize a voltage (or more) on the oscilloscope.

2. Theoretical part:

- The average value of a voltage U(t) (or a current) is given by the following formula:

Umoy=1Tt0t0+TUtdt

T: period of the signal U(t).

- The effective value of a voltage U(t) (or a current) is given by the following formula:

Ueff=1Tt0t0+TUt²dt

T: period of the signal U(t).

3. Course question:

For a signal U(t)=V_max sin⁡(ωt)+B, with: V_max: the amplitude of the signal and ω: pulsation of the signal (ω=2πf).

Calculate the average value U_moy and the effective value U_eff as a function of A. Give the numerical value of U_moy and U_eff for V_max=6V, B=2V and f=100Hz.

| Umoy | Ueff |
| --- | --- |
| …………………………………………………………. ………………………………………. ………………………………………………………….. ………………………………………………………….. …………………………………………………………. ………………………………………………………….. ………………………………………………………...... ………………………………………………………….. …………………………………………………………. N.A. : ………………………………………………. | …………………………………………………………. ………………………………………. ………………………………………………………….. ………………………………………………………….. …………………………………………………………. ………………………………………………………….. ………………………………………………………...... ………………………………………………………….. …………………………………………………………. N.A : ………………………………………………. |

4. Practical part:

Adjust the GBF (Low Frequency Generator) so that it delivers the following signal:

Ut=6sin2π.100.t+2 [V]

| Vmax=………V | Forme=…………………. | Fréquency=……………Hz | B=………………V |
| --- | --- | --- | --- |

Visualize the voltage U(t) across the GBF using the oscilloscope in DC mode and take measurements using the “measure” button on the oscilloscope.

T=………………… […]

F=………………… […]

Vmax=…………… […]

Vmin=…………… […]

Vmoy=…………… […]

Veff=……………… […]

Visualize the voltage U(t) across the GBF using the oscilloscope in AC mode

T=………………… […]

F=………………… […]

Vmax=…………… […]

Vmin=…………… […]

Vmoy=…………… […]

Veff=……………… […]

5. Take measurements with the voltmeter:

Ut=6sin2π.100.t+2                  [V]

Note: We always choose the largest caliber of the value to be measured

Measure the voltage across the GBF using the voltmeter in DC mode

VDC= …………V it represents Umoy  Ueff

Measure the voltage across the GBF using the voltmeter in AC mode

VAC= …………V it represents Umoy  Ueff

complete the following table

|  | Theoretical (Q3) | Oscilloscope(Q4) | Voltmetre (Q5) |
| --- | --- | --- | --- |
| Vmoy | ……………………….. | ……………..…. | ……..……. |
| Veff | …………………………… | ……………..…. | …..………. |

What are your comments……………………………………………………………

6. Take measurements with the ammeter:

Now we will carry out the following experiment : U=5V, R1=1KΩ

| Create the following circuit: | Take the following measurements: U=……………V I=………… mA Calculate : R1 Calculated =U/I=……………KΩ |
| --- | --- |

7. Take measurements with the Ohmmeter:

Using the multimeter, measure the value of the resistance available:................................... Ω

………………………….. Ω

|  | Color Code | Nominal value | Tolerance | value range |
| --- | --- | --- | --- | --- |
| R1 | ………………………… | ……………..….Ω | ……..….Ω | between : …….…..Ω and ……..…Ω |
| R2 | …………………………. | ……………..….Ω | …..…….Ω | between : ………...Ω and…..……Ω |

Comments……………………………..

Noir

Marron

Rouge

Orange

Jaune

Vert

Bleu

Violet

Gris

Blanc